Specialty Manufacturing, Inc.

Technical Bulletin

S67-0001-00

6-Series and 7-Series Stop Arm Wiring Information

This manual contains proprietary information of SMI. It is intended solely for the information and use of parties operating and maintaining the equipment described herein. Such proprietary information may not be used, reproduced, or disclosed to any other parties for any other purpose without the expressed written permission of SMI.

Purpose:  This technical bulletin provides wiring information for use with SMI 6-Series and 7-Series Stop Arms. This information can be used for repair or upgrade work on these units.

Wiring Table

| 6-Series Module |  |  | 7-Series Module |  |  | Bus-side connection |  |  | Relay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pin | Wire Color | Function | Pin | Wire Color | Function | Pin | Wire Color | Function | Pin | Wire Color | Function |
| P4 | Red 2 | +12VDC Motor | 3B | Brown | +12VDC Motor |  |  |  |  |  |  |
| P1 | Red 1 | +12VDC Supply | 2D | red | +12VDC Supply | 2D | red | +12VDC Supply |  |  |  |
| P6 | Blue 1 | +12VDC Trigger | 2F | Blue | +12VDC Trigger | 2F | Blue | +12VDC Trigger |  |  |  |
| P3 | Black | -12VDC Motor | 3A | Yellow | -12VDC Motor |  |  |  |  |  |  |
| P2 | white | Ground | 2E | white | Ground | 2E | white | Ground | 86 | white | Ground |
| P5 | blue 2 | Strobe | 2B | purple | Strobe | 2B | brown | Strobe | 85 | blue | Strobe |
|  |  |  | 2G | Green | Extra Ground | 2G | Black | Extra Ground |  |  |  |
|  |  |  | 2A | n/a | no connection | 2A | n/a | no connection | 87 | n/a | no connection |
|  |  |  | 2C | gray | +12VDC out | 2C | orange | +12VDC out |  |  |  |
|  |  |  |  |  |  |  |  |  | 30 | yellow | Line Out |
|  |  |  |  |  |  |  |  |  | 87a | yellow | Line Out |