INSTRUCTIONS FOR REPLACING FAN HATCH HANDLE ASSEMBLY

FOR 8910B MODEL HATCHES.

- Remove 10-24 button head screw with 1/8” allen wrench and discard screw.

- Remove red handle (with ½” aluminum tubular insert inside), rubber boot, and black cam.  Discard aluminum insert AND rubber boot.

- Using extra deep well ¾” socket, TIGHTEN flange nut.

- Using appropriate e-clip spreader tool, remove e-clip and discard.

- Using ¾” socket, loosen and remove flange nut.  Remove washers (o-ring may be left in place.)

- Pull exterior handle assembly out from top of hatch and discard, replace with new exterior handle assembly.

- From inside bus, replace o-ring (if previously removed), two washers, and flange nut.

- Tighten flange nut with ¾” socket.

- Replace e-clip in groove.  LOOSEN flange nut until flange nut just touches e-clip.

- Replace black cam, ADD NEW ALUMINUM SPACER WITH INDENTS TO FIT CAM AND HANDLE, and red handle (do NOT use ½” aluminum tubular insert).

- Insert new 10-24 button head screw.

Questions?  Call Specialty Manufacturing at 1-800-951-7867